Міністерство освіти і науки  України

Національний університет "Львівська політехніка"

Звіт з лабораторної роботи №3

з дисципліни :«Комп’ютерні системи»

на тему:

“ Аналіз програмної моделі процесу роботи арифметичного конвеєра, ч.1.”

Виконав: студент. гр. КІ-32  
            Янчинський А.В.

Прийняв: викладач
         Козак Н.Б.

Львів – 2020

МЕТА РОБОТИ: Навчитись здійснювати аналіз програмних моделей  комп’ютерних систем, виконаних на мові System C.

Завдання:

- Проаналізувати склад програмної моделі арифметичного конвеєра, (програма PIPE), яка  виконана на мові System C.

- Визначити інформаційні потоки у моделі арифметичного конвеєра.

- Визначити зв’язки керування.

- Накреслити блоки, з яких складається арифметичний конвеєр згідно поданої моделі.

Виконання:

- Проаналізувати склад програмної моделі арифметичного конвеєра, (програма PIPE), яка  виконана на мові System C.

Програмна модель складається з  5 модулів:

- Numgen – модуль який генерує на свої 2 виходи числа.

- Stage1 – модуль обчислює суму та різницю вхідних значень.

- Stage2 – модуль обчислює добуток та частку вхідних значень.

- Stage3 – модуль обчислює значення a в степені b (a та b вхідні сигнали).

- Display – модуль відображає на екрані значення з вхідного порту.

- Визначити інформаційні потоки у моделі арифметичного конвеєра.

Потік даних починається з Numgen далі йде до Stage1, потім з Stage1 йдуть до Stage2, потім Stage2 з йдуть до Stage3, потім дані з Stage3 йдуть до Display.

- Визначити зв’язки керування.

Зв’язки керування виглядають наступним чином:

- Модуль Numgen впливає на Stage1.

- Модуль Stage1 залежить від Numgen, та впливає на Stage2.

- Модуль Stage2 залежить від Stage1, та впливає на Stage3.

- Модуль Stage3 залежить від Stage2, та впливає на Display.

- Модуль Display залежить від Stage3.

- Накреслити блоки, з яких складається арифметичний конвеєр згідно поданої моделі.

Рис. 1 Модель ариф. кон.

Лістинг програми:

File main.cpp

#include "systemc.h"

#include "stage1.h"

#include "stage2.h"

#include "stage3.h"

#include "display.h"

#include "numgen.h"

int sc_main(int ac, char *av[])

{

//Signals

sc_signal<double> in1;

sc_signal<double> in2;

sc_signal<double> sum;

sc_signal<double> diff;

sc_signal<double> prod;

sc_signal<double> quot;

sc_signal<double> powr;

//Clock

sc_signal<bool>   clk;

numgen N("numgen");               //instance of `numgen' module

N(in1, in2, clk );                //Positional port binding

stage1 S1("stage1");              //instance of `stage1' module

//Named port binding

S1.in1(in1);

S1.in2(in2);

S1.sum(sum);

S1.diff(diff);

S1.clk(clk);

stage2 S2("stage2");              //instance of `stage2' module

S2(sum, diff, prod, quot, clk );  //Positional port binding

stage3 S3("stage3");              //instance of `stage3' module

S3( prod, quot, powr, clk);       //Positional port binding

display D("display");             //instance of `display' module

D(powr, clk);                     //Positional port binding

sc_start(0, SC_NS);               //Initialize simulation

for(int i = 0; i < 50; i++){

clk.write(1);

sc_start( 10, SC_NS );

clk.write(0);

sc_start( 10, SC_NS );

}

return 0;

}

File display.h

#ifndef DISPLAY_H

#define DISPLAY_H

struct display : sc_module {

sc_in<double> in;        // input port 1

sc_in<bool>   clk;       // clock

void print_result();     // method to display input port values

//Constructor

SC_CTOR( display ) {

SC_METHOD( print_result ); // declare print_result as SC_METHOD and

dont_initialize();

sensitive << clk.pos();    // make it sensitive to positive clock edge

}

};

#endif

File display.cpp

#include "systemc.h"

#include "display.h"

#include <stdio.h>

//Definition of print_result method

void display::print_result()

{

printf("Result = %f\n", in.read());

} // end of print method

File numgen.h

#ifndef NUMGEN_H

#define NUMGEN_H

struct numgen : sc_module {

sc_out<double> out1;      //output 1

sc_out<double> out2;      //output 2

sc_in<bool>    clk;       //clock

// method to write values to the output ports

void generate();

//Constructor

SC_CTOR( numgen ) {

SC_METHOD( generate );   //Declare generate as SC_METHOD and

dont_initialize();

sensitive << clk.pos();  //make it sensitive to positive clock edge

}

};

#endif

File numgen.cpp

#include "systemc.h"

#include "numgen.h"

// definition of the `generate' method

void numgen::generate()

{

static double a = 134.56;

static double b = 98.24;

a -= 1.5;

b -= 2.8;

out1.write(a);

out2.write(b);

} // end of `generate' method

File stage1.h

#ifndef STAGE1_H

#define STAGE1_H

struct stage1 : sc_module {

sc_in<double> in1;   //input 1

sc_in<double> in2;   //input 2

sc_out<double> sum;  //output 1

sc_out<double> diff; //output 2

sc_in<bool>    clk;  //clock

void addsub();       //method implementing functionality

//Counstructor

SC_CTOR( stage1 ) {

SC_METHOD( addsub );     //Declare addsub as SC_METHOD and

dont_initialize();

sensitive << clk.pos();  //make it sensitive to positive clock edge

}

};File stage1.cpp

#include "systemc.h"

#include "stage1.h"

//Definition of addsub method

void stage1::addsub()

{

double a;

double b;

a = in1.read();

b = in2.read();

sum.write(a+b);

diff.write(a-b);

} // end of addsub method

File stage2.h

#ifndef STAGE2_H

#define STAGE2_H

struct stage2 : sc_module {

sc_in<double>  sum;      //input port 1

sc_in<double>  diff;     //input port 2

sc_out<double> prod;     //output port 1

sc_out<double> quot;     //output port 2

sc_in<bool>    clk;      //clock

void multdiv();          //method providing functionality

//Constructor

SC_CTOR( stage2 ) {

SC_METHOD( multdiv );     //Declare multdiv as SC_METHOD and

dont_initialize();

sensitive << clk.pos();   //make it sensitive to positive clock edge.

}

};

#endif

File stage2.cpp

#include "systemc.h"

#include "stage2.h"

//definition of multdiv method

void stage2::multdiv()

{

double a;

double b;

a = sum.read();

b = diff.read();

if( b == 0 )

b = 5.0;

prod.write(a*b);

quot.write(a/b);

} // end of multdiv

File stage3.h

#ifndef STAGE3_H

#define STAGE3_H

struct stage3: sc_module {

sc_in<double>  prod;     //input port 1

sc_in<double>  quot;     //input port 2

sc_out<double> powr;     //output port 1

sc_in<bool>    clk;      //clock

void power();            //method implementing functionality

//Constructor

SC_CTOR( stage3 ){

SC_METHOD( power );      //declare power as SC_METHOD and

dont_initialize();

sensitive << clk.pos();  //make it sensitive to positive clock edge

}

};

#endif

File stage3.cpp

#include <math.h>

#include "systemc.h"

#include "stage3.h"

//Definition of power method

void stage3::power()

{

double a;

double b;

double c;

a = prod.read();

b = quot.read();

c = (a>0 && b>0)? pow(a, b) : 0.;

powr.write(c);

} // end of power method

Рис.2 Результат роботи програми

Рис.3 Часова діаграма роботи програми

Висновок: на даній лабораторній роботі навчився здійснювати аналіз програмних моделей комп’ютерних систем, виконаних на мові System C.